Haws RIVIVE™ Hydration Station™

Guide Specification

Plumbing and safety environments are evolving, and Haws is redefining expectations. Just as we have done for more than 100 years. From the on-wall bottle filling station and water cooler to the in-wall bottle filling station and fountain, to the outdoor pedestal mounted adjustable series bottle filling stations, Haws is there for all your needs.

The RIVIVE architectural series in-wall bottle filling stations offers esthetically unique constructed lines for the concerning customer while at the same time delivering the tough, vandal resistant stainless-steel construction needed for schools, airports, and other high traffic areas.

Every day we work to address your needs and solve your challenges by applying imagination and initiative to make your job easier. Haws invents, manufactures and builds drinking fountains and standardized and customized emergency response products. With more than 8,000 distribution locations and 250 employees worldwide, we continually focus on quality, service, reliability and complete solution support.

Haws is the only manufacturer who provides warranty and field services for emergency shower and eye wash products of all brands to ensure your equipment is functioning properly and meets ANSI/ISEA Z358.1 compliance.

Contact Haws Corporation, Sparks, NV 89436; Phone: 1-888-640-4297 or visit the Haws web site www.hawsco.com

SECTION 22 47 13 & 22 47 16 - DRINKING

FOUNTAINS/BOTTLE FILLERS & WATER COOLERS

- GENERAL

      - RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - Section includes:

          - Haws RIVIVE™ Hydration Station™ drinking fountain/bottle filler combinations.

          - Haws RIVIVE™ Hydration Station™ bottle fillers.

      - ACTION SUBMITTALS

        - Product Data: For each type of drinking fountain.

          - Include construction details, material descriptions, dimensions of individual components and profiles, and finishes.

          - Include operating characteristics, and furnished specialties and accessories.

        - Sustainable Design Submittals:

          - Product Data: For water consumption.

      - CLOSEOUT SUBMITTALS

        - Maintenance Data: For drinking fountains to be included in maintenance manuals.

      - MAINTENANCE MATERIAL SUBMITTALS

        - Furnish extra materials that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

          - Filter Cartridges: Equal to <Insert number> percent of quantity installed for each type and size indicated, but no fewer than <Insert number> of each.

      - WARRANTY

When warranties are required, verify with Owner's counsel that special warranties stated in this article are not less than remedies available to Owner under prevailing local laws. Coordinate with Section 016000 "Product Requirements."

        - Special Warranty: Manufacturer's standard form in which manufacturer agrees to repair or replace components of units that fail in materials or workmanship within specified warranty period.

Verify available warranties and warranty periods for units and components with manufacturers listed in Part 2.

          - Warranty Period:

            - For Drinking Fountains: One year from date of Substantial Completion.

            - For Bottle Fillers: One year from date of Substantial Completion.

            - For product with a chiller: Additional four years from date of Substantial Completion on pressurized refrigeration system.

- PRODUCTS

      - DRINKING FOUNTAINS

Copy "Drinking Fountains" Paragraph below and re-edit for each type of drinking fountain required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each drinking fountain.

        - Bottle filling stations and drinking fountain(s)/bottle filler combinations.

          - Basis-of-Design Product: Subject to compliance with requirements, provide; [1001HS] [1001HSA] [1001HSA.8] [1011HS] [1011HSA] [1011HSA.8] [2000HS] [2000HSSM] [2000HSA.8] by Haws Corporation, Sparks, NV 89431; (888) 640-4297; fax (775)359-7424; Email customerservice@hawsco.com; Website www.hawsco.com.

- a.     Submit requests for substitution in accordance with Instructions to Bidders and Division 01 General Requirements.

          - Standards:

            - Comply with NSF 61 and NSF 372.

            - Comply with ICC A117.1.

          - Fountain and/or bottle filler: [Bottle filler only], [Bottle filler/drinking fountain combination].

          - Receptor(s):

            - Material: 14-gauge stainless steel wall bracket, 18-gauge stainless steel receptor(s) and bottle filler alcove.

            - Bubbler: One for each receptor, with adjustable stream regulator.

            - Drain: Grid style – fountain and bottle filler.

          - Water Flow: 0.5 gpm (1.89 L/min) for the fountain, and 1.1 gpm (4.16 L/min) laminar flow for the bottle filler, with 100% lead free waterways.

          - Filtration: 3000-gallon (11356 L) capacity, Electronic Lifecycle Control filter cartridge with filter replacement indicator light, and field-configurable as either filtered or non-filtered.

          - Controls: Push-button actuated stainless steel body cartridge style valve with front access to stream adjustment and water supply strainer on the fountain, and sensor operation on the bottle filler.

          - Access to Internal Components: 1001HSA, 1001HSA.8, 1011HSA, 1011HSA.8, 2000HS, 2000HSSM, 2000HSA.8.

          - Energy Efficient: Smart energy savings feature powers down refrigeration system after two hours of non-use and will automatically start again when used (on chilled models).

          - Auto purge: Auto purge feature automatically cycles once every 24 hours of no use to assure a fresh water supply, as well as trap integrity.

          - Programmable: Front access for changing smart filter incorporates easy set, change and monitor reset programming

          - Vandal Resistance: Vandal resistant stainless-steel construction and bottle filler sensor with a 20-second timeout function which prevents unwanted use, and resistant to scratching and water droplet fowling.

          - Supply Piping: NPS 3/8 (DN 10) compression on fountain and bottle filler.

          - Drain Piping: NPS 1-1/4 (DN 32) waste pipe on the fountain and bottle filler.

          - Drinking Fountain Mounting Height: [Standard] [Disabled accessible according to ICC A117.1] [Child-height for children's use].

          - Chilled Water Supply:

            - Environmentally safe R134a refrigerant.

            - Compressor:

              - Air cooled.

              - Electrical Characteristics:

                - Volts: 115V.

                - Phase: Single.

                - Amps: 5 amp rating.

                - 1/5HP.

              - Certification: ANSI/NSF Standard 61 and NSF 61 Section 9, ARI Standard 1010 and ANSI A117.1 Chilled water lines to be insulated.

          - Support: in-wall mounting frame with integral 3/16-inch (4.75-mm-) thick fixture mounting plate

      - SUPPORTS

        - Type I Water Cooler Carrier:

          - Basis-of-Design Product: Subject to compliance with requirements, provide Haws Corporation; [6800] [6810] or comparable product

          - Standard: ASME A112.6.1M.

- EXECUTION

      - EXAMINATION

        - Examine roughing-in for water-supply and sanitary drainage and vent piping systems to verify actual locations of piping connections before fixture installation.

        - Examine walls and floors for suitable conditions where fixtures will be installed.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - INSTALLATION

        - Install fixtures level and plumb according to roughing-in drawings. For fixtures indicated for children, install at height required by authorities having jurisdiction.

        - Install recessed drinking fountains secured to rigid framing in wall construction.

        - Install off-the-floor carrier supports, affixed to building substrate, for wall-mounted fixtures.

        - Install water-supply piping with shutoff valve on supply to each fixture to be connected to domestic-water distribution piping. Use ball or gate valve. Install valves in locations where they can be easily reached for operation. Valves are specified in Section 220523.12 "Ball Valves for Plumbing Piping" and Secti*-on 220523.15 "Gate Valves for Plumbing Piping."

        - Install trap and waste piping on drain outlet of each fixture to be connected to sanitary drainage system.

        - Insulate all water supply lines subject to condensation.

        - Seal joints between fixtures and walls using sanitary-type, one-part, mildew-resistant, silicone sealant. Comply with sealant requirements specified in Section 079200 "Joint Sealants."

      - CONNECTIONS

Coordinate piping installations and specialty arrangements with Drawings and with requirements specified in piping systems. If Drawings are explicit enough, these requirements may be reduced or omitted.

        - Connect fixtures with water supplies, stops, and risers, and with traps, waste, and vent piping. Use size fittings required to match fixtures.

      - ADJUSTING

        - Adjust fixture flow regulators for proper flow and stream height.

      - CLEANING

        - After installing fixtures, inspect unit. Remove paint splatters and other spots, dirt, and debris. Repair damaged finish to match original finish.

        - Clean fixtures, on completion of installation, according to manufacturer's written instructions.

        - Provide protective covering for installed fixtures.

        - Do not allow use of fixtures for temporary facilities unless approved in writing by Owner.

END OF SECTION 22 47 13 & 22 47 16